grabmode, a text/XWindows mode grabber

======================================

Another tool to get SVGAtextMode up and running. While the clock probe just

tells you what pixel clock is actually in use, this program grabs the entire

text mode line from the VGA card, and shows it on the screen in a format

compatible with the TextConfig file.

Syntax:

../grabmode: version 0.9. (c) 1995 Koen Gadeyne.

Usage: ../grabmode [options]

Options: -n  Don't program VGA hardware

-d  print debugging information

-X  Output compatible with XF86Config mode line

-T  Output compatible with TextConfig mode line

-b  Also print blanking information

-c  Don't probe for pixel clock (just print timing information)

-h  print usage information

Grabmode can give you the mode line in two formats: one compatible with

SVGAtextMode, and one compatible with XF86Config.

Normally grabmode is used WITHOUT the -T or -X argument. It finds out from

the VGA hardware whether you are running it from text mode or graphics mode,

and adjusts its output accordingly.

When executed from the same text mode, you get the following results

(example):

./grabmode -T  (or just "grabmode", since this IS text mode)

./grabmode: Please be patient. This may take a while (up to 1 minute)

"132x64"   75.165   1056 1088 1240 1336   768 775 776 800   +Hsync +Vsync  font 8x12   # 56.262kHz/70.33Hz

./grabmode -X

./grabmode: Please be patient. This may take a while (up to 1 minute)

"1056x768"   75.167   1056 1088 1240 1336   768 775 776 800   +Hsync +Vsync  # 56.263kHz/70.33Hz

A word of warning: If you use "grabmode -T" from a graphical screen (like

from X-Windows) it will give rather strange results. The example below was

executed from a 1024x768 X-windows screen:

"128x768"   75.165   1024 1064 1216 1336   768 775 776 800   +Hsync +Vsync  font 8x1   # 56.261kHz/70.33Hz

While "grabmode -X" would read (correctly):

"1024x768"   75.165   1024 1064 1216 1336   768 775 776 800   +Hsync +Vsync  # 56.261kHz/70.33Hz

Since graphical screens have nothing to do with hardware assisted font

rendering (as from VGA text mode - let's forget about accelerators for a

while), they set the "character height" to 1 pixel (in fact they have to,

otherwise each line gets repeated 16 times, if they leave the "16" from the

9x16 character mode in there!).

If you still want to do this, e.g. in order to grab the X-Windows timings to

use them in TextConfig, you'll have to adapt the width and height of the

extracted mode, and add an appropriate "font ..." line to it. grabmode

cannot do this conversion for you, since it doesn't know what font size you

want.

You can use this tool either to check the current mode (if something went

wrong, you want to know what REALLY got programmed in that slap of silicon

they call the "VGA chip"), or to grab an existing text mode and import it

into SVGAtextMode.

And to enable you to grab the text modes from DOS, Kenneth Albanowski has

prepared a DOS port of grabmode, "grabmode.exe". A clever move! This way you

can get the modes from DOS, if the Linux boot doesn't recognize your VGA

card.

Note that the options for this DOS version must be entered with a "/"

prepended, and not a "-" as in UNIX.

Known problems/limitations:

---------------------------

A special note to Windows users: I would really like to get this thing

to work from Windows, since you could then grab your Wintendo graphics mode,

and import it into your XF86Config file, giving you EXACTLY the same modes

in X as under Windows. A really useful tool for those uncomfortable with

X-configuration.

Alas... It doesn't work  from windows... Windows seems to hide its VGA

registers from normal programs (probably through the MMU address mapping), so

the mode grabber gets stuck trying to determine the pixel clock. I'd be very

interested in any comments and/or suggestions on how this can be done.

Grabmode also doesn't correctly detect emulated CGA modes (e.g. 320x200). It

detects a 320x200 mode as being 640x400. When I find out how this is done,

I'll change grabmode to detect and correctly display this situation.

Grabmode will NOT report correct settings on non-256 color modes (15/16 bit,

24 bit). It was designed to debug and/or grab TEXT modes, and the graphics

grabbing is just a bonus, because that was really easy to implement.

BUT if it must be able to detect those special modes correctly, it would

have to know about the VGA chip it is running on (SVGA modes are programmed

differently on each VGA chip), and that would mean VGA chip probing, DAC

type probing, etc... And that is really not my intention. Maybe some VGA

guru will be tempted by grabmode to extend it to those special modes.

To make things even worse, grabmode will not even correctly report simple

256-color (8-bit) modes on some cards. Especially high-end cards (like #9,

Diamond, ...) using special VGA DAC's (Bt485,...) with pixel multiplexing

will report completely wrong things on higher (>67.5 MHz, for example)

pixel clocks.

Enjoy! (for what it's worth...)

extra note: pixel multiplexing, 15/16 bit and 24 bit modes

----------------------------------------------------------

Why doesn't grabmode work correctly on non-256 color graphics modes on most

cards? To understand this, you have to dig a little deeper into the

specifics of VGA cards. If you like tech-talk, read on!

Normal, "el cheapo" VGA cards come with a DAC on it that has an 8-bit

interface to the VGA chip. Even most so-called "24-bit" DAC's have just

8-bit interfaces to the VGA chip. They have a "color palette" lookup table

on board (that makes them a "RAMDAC" instead of just a "DAC") which can

translate an 8-bit value into a 24-bit (or 18-bit on really cheap things)

color value before sending that off to three 8-bit (or six bit) DAC's, and

then to the RGB-outputs of the VGA card.

In "normal" 256-color graphics modes, this is an ideal solution: one pixel

just needs one byte, and one byte is sent to the DAC for each pixel. So when

the pixel rate (not necessarily equal to the pixel CLOCK!) is e.g. 75 MHz

for an 1024x768x256 @ 70Hz graphics mode, the data rate through that channel

from the VGA chip to the DAC is also 75 MHz. And in this case this is of

course also the pixel CLOCK. Very simple.

BUT. When you want higher-resolution images, like 1280x1024x256 @ 70 Hz, the

pixel rate must increase to 125 or even 135 MHz. And most relatively

low-cost CMOS designs can go up to 110 MHz, but in most cases not much

higher (ET4000, Cirrus Logic, Trident,... all stop at 90 MHz, S3-805 for

example has a max of 110 MHz). Going higher than that would make the VGA

chip cost LOTS more. So what did they do?

They used a wider interface, like e.g. 16-bit (eg ET4000W32p), 24-bit (eg

Bt485) or for really high-end stuff: 48, 64, 96, 128, 256-bit (eg IBM,)

interfaces. The DAC then either needs to double (triple, ...) the pixel

clock internally by itself (in the 16-bit case it gets 2 pixels per clock

beat, so it needs to create the double clock to be able to display both

pixels sequentially), or get that clock from the VGA chip.

Creating the actual pixel clock only inside the DAC, instead of externally

generating it and trying to feed it to all other chips that would need it

has some major advantages. Price is of course the most important reason:

high-speed digital boards need a very well though-out layout, are more

difficult to get through those stringent US FCC-rules, can create

power-dissipation problems, and use more expensive components.

All this means you can let the VGA chip run at the "cooler" (and cheaper)

clocks of 1/2, 1/3, ... of the pixel clock. But of course both the VGA chip

and the DAC have to "know" about this.

This also explains the limit on many cheap VGA cards on maximum 15/16 and

24-bit resolutions: since they have only 8-bit interfaces, they need to send

TWO bytes for each 16-bit pixel, and even THREE for 24-bit pixels. And they

have to do that over the same 8-bit bus! Those VGA cards need to be

programmed with the DOUBLE pixel clock for 16-bit modes, and TRIPLE the

pixel clock for 24-bit modes. e.g. an 640x480 mode needs only 25 MHz pixel

clock at 8-bit per pixel. But 50 MHz at 16-bit, and 75 (!) MHz at 24-bit.

My S3-805 card is such an example. I DO have 2MB of video RAM on board, and

that would theoretically allow me to do resolutions OVER 800x600 at 24-bit.

BUT, since 800x600 requires a 50 MHz clock (for 70 Hz refresh) at 8 bit

colors, running it at 24 bit would require 150 MHZ!!! And that would

probably fry it for good...

If you would have a DAC with a "wider" interface, you wouldn't need those

double/triple clocks: you can send a pixel in its whole in one clock beat!

So your card probably CAN handle higher resolutions at 24-bit! Of course,

such a card is a bit more expensive.

Also, most accelerators cannot work at 24-bit, since that is a rather clumsy

memory organisation, and would be hard to implement in silicon. In most

cases, silicon designers choose for a 32 bit pixel format. This waists a

byte per pixel, but is a lot easier in hardware than 24-bit. You migh have

read in the docs for XFREE3.x that 24-bit modes are NOT supported, but

32-bit modes are.

What is the impact of all this on grabmode?

As already mentionned above, grabmode does NOT know about ANY special VGA

chip. It just reads registers, and reports what's in there. So if some chip

needs real values divided by 4 (because it uses pixel multiplexing over a

wider-than-8 bit bus to reduce the external clock rates), then grabmode will

NOT know about this. This is a sad situation, but I didn't want to put all

kinds of chipset-detection code in it. First of all, that's too much work.

Secondly, you're always at least one step behind chip set makers. Thirdly,

grabmode was primarily designed for testing text modes (debugging). I added

graphics modes to it, because that was EASY. At least, for standard chips...

For the moment, I do NOT intend to do anything about that...

Summary: grabmode only works for text modes and "simple" graphics modes :-(